Methodology

The additional energy demand of conventional SMR relative to eSMR was decomposed into (i) feed preheating and reaction enthalpy, and (ii) irrecoverable thermal losses in the firebox and radiant section — comprising flue gas losses (6–10%) and insulation losses (1–5% of fired energy) [Ullmann's]. Under ideal heat integration, contribution (i) is fully recovered as steam export, rendering it economically equivalent to dedicated NG-fired steam generation at sites operating under a net steam deficit. The net differentiator therefore reduces to the irrecoverable thermal losses of conventional SMR.

The electricity-to-natural gas price ratio was evaluated for Germany, the United States, and China using 2019 industrial energy prices as a pre-crisis baseline [5,6], accounting for the fact that the reaction enthalpy — near-identical in both processes — is supplied by different energy carriers: natural gas in conventional SMR and electricity in eSMR. The CO₂ certificate price required to close the residual economic gap was then derived analytically using the natural gas emission factor EFNG = 0.205 t CO₂/MWhNG [Umweltbundesamt, 2022]:

This threshold was compared against current EU ETS carbon price levels to assess whether CO₂ pricing materially alters the conclusions.

The additional energy demand of post-combustion CO₂ capture relative to pre-combustion capture was derived from the techno-economic simulation results reported by IEAGHG (2017) for an SMR-based merchant hydrogen plant. Two energy components were quantified separately: the thermal energy required for solvent regeneration in the stripper reboiler, and the electrical energy consumed by the capture and compression units, including auxiliary loads. The reboiler duty was calculated directly from the reported LP steam consumption of the CO₂ capture plant (64.4 t/h for pre-combustion MDEA, 96.4 t/h for post-combustion MEA) multiplied by the specific enthalpy difference between LP steam (0.44 MPa, 177 °C; h ≈ 2810 kJ/kg) and saturated condensate return (0.44 MPa, 147 °C; h ≈ 620 kJ/kg) from NIST steam tables. The electrical penalty was determined as the sum of differential power consumption of the hydrogen plant, COGEN auxiliaries/BoP, CO₂ capture plant, and CO₂ compression and drying unit, taken directly from the simulation results. Both energy components were normalized to the natural gas feedstock energy input (LHV basis) to enable transferability to other applications. To express both energy forms on a common primary-energy basis, the reboiler heat was divided by a boiler efficiency of 0.95 to account for the additional fuel required in a standalone installation, while the electrical demand was converted to primary energy using a factor of 2.5, corresponding to an average grid generation efficiency of approximately 40%. The incremental energy penalty of post-combustion over pre-combustion capture is reported as the sum of both contributions, expressed as a fraction of the natural gas feedstock energy input on an LHV basis to allow comparison across applications.

The energetic effect of high-pressure operation was assessed by comparing the pressure work associated with conventional pressure reduction of the natural gas feed and subsequent recompression of the hydrogen product. Both steps were treated as ideal-gas, isentropic processes corrected by representative isentropic machine efficiencies. The isentropic exponents were calculated from temperature-dependent heat capacities according to γ=cp/(cp-R), using cp values for methane and hydrogen at the selected reference temperature, resulting in γCH4≈1.30 and γH2≈1.41. The recoverable expansion work of the natural gas stream and the required hydrogen compression work were calculated from the molar-flow-based isentropic work expressions, with real-machine corrections WE,real=ηEWE,is and WC,real=WC,is/ηC. A simplified value of ηE=ηC=0.85 was assumed for both expansion and compression. The resulting net pressure-work balance was defined as ΔWE-C=WE-WC, where negative values indicate that the avoided downstream hydrogen recompression demand exceeds the recoverable expansion work from natural gas pressure reduction. All energy terms were finally converted to primary energy and normalized to the natural gas feedstock energy analog to above.

To assess the decarbonization potential of the eSMR concept independently of regional energy price distortions, the abatement effectiveness – defined as t CO₂ avoided per MWh of renewable electricity input – was evaluated under theoretical upper-bound conditions. The metric was derived from the ratio of avoided NG-fired heating emissions to the electrical energy required to drive the reforming reaction. Since the eSMR concept eliminates the steam co-product through internal heat integration, the equivalent steam demand at integrated chemical sites must alternatively be supplied via electrified heat generation, contributing an additional abatement effectiveness

Comparison to Conventional SMR Process

For SMR-based hydrogen production from natural gas (NG), capital expenditures (CAPEX) is of secondary importance to economic viability, as operating expenditures (OPEX) – dominated by NG feedstock costs (45-75% of total production costs [1]) – constitutes the primary cost driver. Including CCUS, CAPEX is between 25% in high-fuel-cost-regions like Europe or China and 40% in low-fuel-cost regions like Middle East or US [1]. Especially in Europe it can be as low as 7% according to other studies [2]. This highlights that the CAPEX share depends heavily on the assumptions made.

Consequently, for process configurations of similar plant topology, a block flow diagram level assessment is sufficient to draw first-order conclusions on economic competitiveness, since variations in equipment layout and capital intensity are unlikely to alter the fundamental dominance of OPEX.

Comparison of the electrified SMR (eSMR) process with the conventional process at the block flow diagram level reveals that NG energy is substituted by electrical energy, without a significant reduction in overall energy consumption. Given that electricity prices are substantially higher than NG prices, this substitution does not yield an economically viable alternative to the incumbent process.

To elaborate, the individual process concepts are examined step by step, beginning with the pressure swing adsorption (PSA) unit and its off-gas integration, before tracing the full process sequence chronologically.

- Both concepts employ a PSA for final product purification, yielding pure hydrogen as the product and directing all remaining components to the off-gas. In the conventional process, this H₂-rich off-gas is combusted in the reformer furnace. In the eSMR concept, a purge stream is required to remove nitrogen introduced with the feed. The purge fraction is sized to satisfy the process steam demand internally, with the remainder recycled to the reactor to maximize H2-yield.

- Both processes perform the same reaction set to produce the target product. Consequently, both use NG and steam as feed, with the eSMR additionally incorporating the aforementioned recycle stream.

- The reformer outlet temperature is 850 °C and 1100 °C, respectively. Both exceed the material limitations of standard heat exchangers (500-600 °C [https://doi.org/10.1016/j.rser.2023.113467]), precluding direct feed preheating via the product stream. In the conventional process, the final 200-300 °C of temperature rise is therefore achieved by NG firing, and the product stream is instead used for steam generation – both for internal process use and export. The eSMR reactor concept, by contrast, enables internal heat integration, allowing heat exchange between product and feed streams even at these elevated temperatures.

- Both concepts include a water-gas shift (WGS) reaction step to maximize hydrogen yield.

- In the electrified process, carbon is removed from the cycle via the purge stream; as carbon enrichment is excessive, an additional carbon capture unit is required.

Heating Energy Dynamics and Economic Implications of eSMR vs. Conventional SMR

The key distinction reduces to fuel (NG) demand of the conventional process, attributable to preheating and reaction enthalpy supply, whereas this is provided by electricity in the novel technology.
Since the reaction enthalpy for the target product is almost identical in both processes, the additional energy demand of the conventional process is due solely to preheating, plus losses (flue gas losses and insulation losses in the firebox and radiant section, 6-10% and 1-5% of fired energy, respectively [Ullmann’s chapter as source]). 
Under ideal heat integration, the additional preheating energy input is fully (plus/minus minor ∆cpchange) recovered as steam, generating a steam export. It is worth noting that typical integrated chemical sites generally operate with a net steam deficit [3], which is conventionally offset by NG-fired steam generation. From this perspective, whether steam is produced from NG directly or as a byproduct of the SMR process is equivalent, neglecting minor differences in conversion efficiency. The only true differentiator is therefore the additional losses from flue gas and, potentially, insulation (due to the smaller reactor design in the eSMR concept). Even assuming an ideal, lossless eSMR process, no economic advantage is achieved, given electricity prices are approximately ~250% higher than NG prices corresponding to Germany in pre-crisis times (2019) [3]. In other European countries this correlation is less pronounced, but still significant. This ratio is subject to significant fluctuations and is being exacerbated by geopolitical crises [4]. In other regions the economics differ, yet the price differential remains more dominant than additional losses.

| Region | Final lectricity prices in energy-intensive industries (2019), USD/MWh [5] | Natural gas prices for industry end-user (2019), USD/MWh [6] | Ratio |
| --- | --- | --- | --- |
| Germany | ~70 | 28-30 | ~2.5x |
| United States | ~45 | 11-15 | ~3-4.1x |
| China | ~551 | 31-53 | ~1-1.8x |

But it becomes apparent, that China represents a notable exception, with an electricity-to-gas price ratio significantly below that of the United States and Europe; explainable by low electricity prices resulting from coal and renewable energy [7], coupled with a limited domestic supply of natural gas [8][9]. Making China the most near-term favorable market for eSMR deployment from an operational cost perspective. It must be acknowledged, however, that deploying an electrified process on a carbon-intensive grid [10] does not inherently improve sustainability outcomes.

The preceding analysis has not yet accounted for the CO₂ certificate cost on NG combustion emissions, while the electricity price already incorporates the CO₂ penalty implicitly. After partially offsetting the ~150% electricity-to-NG price gap through the additional thermal losses of conventional SMR (~15%), the CO₂ penalty required to fully close the residual ~135% gap corresponds to a carbon price more than 6 times the prevailing NG price:

Carbon PriceUSD CO2t CO2=NG PriceUSD NGMWh NG∙GapUSD CO2USD NGEFNGt CO2MWh NG↔Carbon PriceNatural Gas Price=135%0.205=6.6

Well beyond current and near-term projected EU ETS price levels, the CO₂ penalty therefore does not materially alter the conclusions drawn above.

EF source: [https://www.umweltbundesamt.de/system/files/medien/479/publikationen/cc_29-2022_emission-factors-fossil-fuels.pdf]

Comparative Assessment of Post-, Pre- and Oxyfuel Combustion CO₂ Capture

Since eSMR produces near-zero direct CO₂ emissions, a fair comparison requires an equally low-carbon reference – namely, conventional SMR equipped with flue gas carbon capture.

In the eSMR process, carbon is removed from a CO₂-rich stream downstream of the shift reactor at high pressures (pre-combustion capture). Assuming complete conversion in both the reformer and shift reactor, the maximum achievable CO₂ content is 20%, as dictated by the combined SMR and WGS stoichiometry:
CH4+2 H2O→CO2+4 H2

xCO2,max=11+4=20%

Although the CO₂ content of a stoichiometric CH4 combustion flue gas of maximal ~12% is notably lower,

CH4+2 O2+2∙0.790.21N2→CO2+2 H2O+7.52 N2

xCO2,max=11+7.52=11.7%

the stoichiometric process emissions are also included due to PSA off-gas combustion.This leads to CO₂ concentrations of e.g. 21% [11] at atmospheric pressure (post-combustion capture):

From the associated CO2 partial pressures follows that carbon capture integrated into the eSMR concept is associated with lower energetic effort and thus cost than flue gas carbon capture. Primarily due to the reduced absolute quantity of CO₂ requiring separation, but also owing to enhanced absorption efficiency at higher partial pressures.

Based on the energy balance derived from a techno-economic evaluation of post- and pre-combustion CO₂ capture [11], it can be concluded that the higher costs associated with post-combustion capture narrow the cost gap by 11% (see Appendix 1). As the evaluated pre-combustion case operates at 25 bar – below the feasible pressure range of the OCMC concept – its thermodynamic efficiency is reduced, but even under the idealized borderline case (pre-combustion capturing costs of 0), the resulting 25% differential remains insufficient to close the gap between electricity and natural gas costs.

The CO₂ loading capacity of chemical and physical absorption solvents intersects at approximately 10 bar CO₂ partial pressure – the primary thermodynamic driver for capture efficiency. Below this threshold, chemical absorption is generally preferred, while physical absorption becomes more favorable at system pressures above ~50 bar. Only beyond this threshold do differing capture methods warrant consideration in a techno-economic comparison.

Another CCS-related separation technology is oxyfuel combustion [Ullmann’s Carbon Dioxide, S.27], which shows better efficiency than post- and pre-combustion at high capture rates (>92%) [https://doi.org/10.1016/j.jece.2021.105777; https://doi.org/10.1021/acsomega.3c05034]. This technology, by contrast, yields a nearly pure CO₂ flue gas with minimal separation effort, but has not reached commercial maturity due to unfavorable economics; mainly because of additional air separation unit (ASU) and compression and purification unit (CPU) [https://doi.org/10.1016/j.fuel.2021.122057].

In retrofit-oriented low-temperature designs, the diluent N₂ of combustion air is replaced by recycled CO₂-rich flue gas, maintaining peak flame temperatures comparable to conventional air-fired operation. High-temperature designs, by contrast, operate without flue gas recirculation, resulting in adiabatic flame temperatures exceeding 2,700 °C [https://papers.ssrn.com/sol3/papers.cfm?abstract_id=4547389] – an increase of approximately 800 K – which surpasses the material tolerance of the OCMC reactor concept. Moreover, this approach does not leverage the innovative feature of the OCMC design, namely the electrical energy input. For those reasons, oxy-fuel combustion is not considered for further investigation as a use case.

Pressure as a Design Parameter: Compression Penalty of Conventional SMR vs. Pressure-Independent eSMR

Conventional SMR operates at 3-3.5 MPa and typical operating temperatures of 850-900 °C [http://dx.doi.org/10.1016/B978-0-08-100116-5.00004-1], with current materials being pushed close to their design limits at 4 MPa [12]. Operating at the natural gas supply pressures >4 MPa is not feasible because tube hoop stress σ = P·D/(2t) would exceed the ~30 MPa creep design stress of current Ni-Cr alloys (HP-Micro) at 100,000 h service life [API STD 530, Guglielmino et al., 2016].

Operating the SMR at elevated pressure offers several process advantages:

- Feed compression prior to the mole-increasing reforming reaction is energetically more favourable than downstream product compression, [source!]

W=n∙γγ-1∙R∙T1∙p2p1γ-1γ-1 with nfeed<nprod

- Increased volumetric throughput, enabling more compact reactor designs.

- Higher partial pressures enhance reaction kinetics, reducing the required reactor volume.

- No impact on mole-neutral WGSR under ideal-gas assumptions.

- Increased theoretical maximum H₂ recovery in PSA, [source!]

Thermodynamic condition: ppollut.ads≥ppollut.des→φmax=1-pdespadsyH20

But increasing thickness of wall tubes is economically limited by [12]

- increased material costs,

- decreased production capacity due to less catalyst volume, and

- deteriorated heat transfer due to higher thermal resistance.

The only downside of elevated pressure – an unfavourable equilibrium shift of the mole-increasing reforming reaction per Le Chatelier's principle – can be offset by increasing the reaction temperature, leveraging the endothermic nature of the reforming reaction to maintain high conversion.

In conclusion, it can be said that there is an interest in operating at high pressures if material and temperature limitations can be overcome.

Natural gas is transported in the German transmission network at pressures of up to 100 bar [https://fnb-gas.de/erdgastransport/fernleitungsnetz/], making high-pressure eSMR operation compatible with direct pipeline supply. Refinery units operate at pressures of 20-200 bar for hydrotreating and 60-200 bar for hydrocracking [https://doi.org/10.1007/978-0-387-25789-1_7] requiring substantial compression of the hydrogen supply after conventional SMR.

Pressure reduction to SMR operating conditions requires an expansion step, followed by recompression for downstream processes, while a pressure-independent reforming concept, such as OCMC, would eliminate both steps:

ideal: γCH4=cpcv=cp(T1)cp(T1)-R=35.69Jmol K35.69-8.314Jmol K=1.30→γγ-1=4.33, γ-1γ=0.23

ideal: γH2=cpcv=cp(T1)cp(T1)-R=28.84Jmol K28.84-8.314Jmol K=1.41→γγ-1=3.44, γ-1γ=0.29

∆WE-C=WE-WC=R∙T1∙γγ-1∙nE∙1-p2p1γ-1γ-γγ-1∙nC∙p2p1γ-1γ-1=8.314kJkmol K∙273.15 K∙4.33∙1863.5kmolh∙1-37.1 bar70 bar0.23-3.44∙4461.5kmolh∙70 bar25 bar0.29-1=-9.64∙106kJh=-2.68 MWh/h

The electrical demand was converted to primary energy using the same factor of 2.5 as above, assuming an efficiency factor of 0.85 [source!] for both (expansion and compression machine) and normalization to 339 MWh/h NG feed results in extra energy costs of 2.3% due to compression. Insufficient to close the gap between electricity and natural gas costs.

Abatement Effectiveness of eSMR Reformer

The long-term decarbonization of industrial processes fundamentally requires electrification powered by renewable electricity. In this context, eSMR is structurally well-positioned: as grids decarbonize and renewable electricity costs decline, its OPEX penalty diminishes and its carbon footprint improves passively with the grid mix. 
The ratio of t CO₂ abated per MWh of renewable electricity is a common metric for assessing the decarbonization impact of electrification technologies, regardless of energy price distortions. Under theoretical upper bound assumptions an Abatement Effectiveness of up to 0.24 t CO2/MWh is achievable:

Abatement Effectiveness=∆HR,NG(850°C)ηfired heater ∙EFNG ∆HR,el(1100°C)=215.1kJmol CH4 reacted0.85∙57t CO2TJ∙10-6TJMJ∙3600MJMWh220.3kJmol CH4 reacted=0.24t CO2MWh

EF source: [https://www.umweltbundesamt.de/system/files/medien/479/publikationen/cc_29-2022_emission-factors-fossil-fuels.pdf]

Additionally, this concept gives the opportunity to electrify the production of the otherwise exported steam since no excess steam is produced as co-product (due to reactor internal heat integration). Power to heat is associated with an Abatement Effectiveness of 0.2t CO2MWh [source!].
However, at current electricity-to-gas price ratios and grid carbon intensities, eSMR remains economically disadvantaged and does not yet deliver genuinely low-carbon hydrogen in most geographic contexts.

Conclusion

This analysis concludes that electrified SMR (eSMR) is unlikely to displace conventional SMR under current economic conditions. The fundamental barrier is the electricity-to-natural gas price differential, which cannot be offset by the environmental benefits of avoided combustion emissions at prevailing or near-term projected carbon prices.

A more equitable comparison pairs eSMR against conventional SMR equipped with post-combustion CCS. In this context, the higher CO₂ partial pressure inherent to pre-combustion capture configurations – as in eSMR – confers a clear thermodynamic advantage over post-combustion processes. This advantage is further amplified when post-combustion capture must treat a simple CH₄ combustion-derived flue gas stream, as encountered in ammonia syngas production, rather than a flue gas stream with additional process CO2.

An additional differentiator is the material-limited operating pressure of conventional SMR (~40 bar), whereas the OCMC-based eSMR concept is unconstrained in this regard. Given that natural gas is typically supplied at higher pressures and downstream synthesis processes require elevated pressures, pre-reaction compression is energetically more favourable than post-reaction compression of the mole-increased product stream. While this pressure advantage is real, its magnitude is smaller than the economic effects analysed above.

Taken together, these considerations suggest that ammonia syngas production represents a more promising near-term application for the OCMC reactor concept than hydrogen-only SMR. This assessment is motivated by three compounding factors: (i) more efficient carbon capturing conditions, (ii) the synthesis gas must be compressed to at least 150 bar for ammonia synthesis, amplifying the pre-reaction compression benefit; (iii) the reactor concept enables high reaction temperatures (>1000 °C) with external electrical energy input, an advantage not accessible in conventional adiabatic reactor configurations, which allows combining two reactors into one and hence a potential for CAPEX reduction without fundamental process redesign (as the process design otherwise remains largely unchanged, relying on commercially proven downstream technologies).

A comprehensive techno-economic analysis (TEA) of the ammonia syngas case would allow quantitative evaluation of the compounding benefits – improved CO₂ capture efficiency, reduced compression costs, and potential CAPEX savings – against the persistent electricity cost penalty. While TEA studies on eSMR for hydrogen production have reported a wide range of outcomes depending on electricity prices and carbon pricing assumptions [e.g., Song et al., 2022; Wismann et al., 2022; Do et al., 2023], dedicated assessments for electrified ammonia syngas production remain notably scarce.

Sources:

[1] https://iea.blob.core.windows.net/assets/9e3a3493-b9a6-4b7d-b499-7ca48e357561/The_Future_of_Hydrogen.pdf

[2] https://www.sintef.no/globalassets/project/elegancy/deliverables/elegancy_d5.2.2_rotterdam_h2-production_integration.pdf

[3] https://onlinelibrary.wiley.com/doi/epdf/10.1002/cite.202100164

[4] https://ieefa.org/resources/europes-electricity-prices-are-still-tied-gas-making-geopolitics-structural-vulnerability

[5] https://www.iea.org/data-and-statistics/charts/estimated-final-electricity-price-for-large-industrial-customers-in-energy-intensive-industries-2019-2024

[6] https://www.iea.org/data-and-statistics/charts/industry-end-user-prices-for-natural-gas-in-selected-countries-2019-march-2023

[7] https://www.iea.org/countries/china/energy-mix

[8] https://ieefa.org/resources/understanding-competitive-landscape-chinas-lng-market

[9] https://www.sciencedirect.com/science/article/pii/S0301421521002500

[10] https://www.iea.org/reports/electricity-2025/emissions

[11] https://ieaghg.org/publications/techno-economic-evaluation-of-smr-based-standalone-merchant-hydrogen-plant-with-ccs/

[12] https://doi.org/10.1177/02670836241262199

Appendix:

First draft:

CAPEX vs OPEX

eSMR vs SMR

Comparison of the SMR process as an electrified process with the conventinal process on a block flow diagramm level, leads to the result, that energy delivered by natural gas is replaced by electrical energy, but energy consumption itself is not reduced significantly. This brings us straigthforwardly to the conclusion, that due to energy prices being on a way higher level than NG prices, this process can’t be implemented economically more viable than the incumbent proess.

To explain that in further depth, let’s have a look into the process concepts step by step, starting with the PSA and its off gas integration, before go through the process downstreamly.

- Both concepts use a PSA for final product purification, resulting in pure hydrogen as product and all other molecules in the off gas. The incumbent process fires the H2-rich off gas in the reformer, while in the eSMR concept a purge is necessary to get rid of the nitrogen introduced with our feed. The recycle is choosen to be just large enough to produce the steam required for the process independently, the rest is recycled to the reactor to maximize yield.

- Both processes perform the same reaction set to yield the product of interest. Consequently, both feeds exists of natural gas and steam, aditionally the electrified process feeds the mentioned recycle stream.

- The outlet stream leaves the reformer at temperatures of 850°C or 1100°C respectively. Both temperatures are higher than the material constraints of standard heat exchangers (500-600°C), which means feed preheating by our product stream is not possible. In the incumbent process the last 200-300°C temperature increase is therefore reached by NG firing. The product stream is instead used to produce steam, for the process as well as export. On the contrary the eSMR reactor concept allows a reactor internal heat integration, which allows heat integration from product to educt stream even on this high of a temperature level.

- Both concepts include a WGS reaction step for hydrogen yield maximization.

- In the electrified process carbon will be removed from the cycle by the purge, or if the enrichment is too high in that case via an additional carbon capture unit is necessary.

The difference can be broken down to higher NG demand of the incumbent process for preheating and reaction enthalpy reasons, which ultimately is compleately converted to steam (assuming an ideal heat integration) and hence a steam export is created. Reaction enthalpy for the products of interest is the same in both processes. Consequently, the higher NG demand can be attributed solely to preheating, in addition any losses (flue gas losses and insulation losses of the firebox and radiant section, 6-10% of fired energy and 1-5% respectively) must be covered. Practically sorted, it needs to be highlighted, that typical integrated plants have overall a deficit in their steam balance, which is offset by steam production most often from natural gas firing. Concluding from this, that it makes no difference (neglecting minior differences in conversion efficiency) whether steam is produced from NG on purpose or as a byproduct from SMR. Hence, the only difference are higher losses through flue gas and potentially through insulation (due to smaller reactor design). Even if assuming the electrified process as ideal with no losses, a economic competitive edge is not given due to about xxx% higher electricity prices compared to NG.

An additional difference occures, if a carbon capture unit is introduced. In that case carbon is removed from a CO2-rich gas stream downstream of the shift reactor. But even assuming an ideal 100% conversion in the reformer and the shift reactor, the CO2 content cannot exceed 20%, according to the combined SMR and WGS reaction

CH4+2 H2O→CO2+4 H2

This is a similar content as in combustion flue gasses (sources). Which leads to the conclusion, that a carbon capture as part of the electrified process concept is associated with the same effort as the flue gas carbon capture. And from the fact that flue gas carbon capture is not widely commercialised, not even in regions with high carbon taxes (e.g., European Union), it can be concluded that its just not economically viable.

Recycle mostly of CO, total amount of recycle could be reduced if a second low temp shift reactor would be added, but for economics it doesn makes a difference on the level of the current analysis.

Another CCS-related separation technology is oxyfuel combustion [Ullmann’s Carbon Dioxide, S.27], which shows better efficiency than post- and pre-combustion at high capture rates (>92%) [https://doi.org/10.1016/j.jece.2021.105777; https://doi.org/10.1021/acsomega.3c05034]. This technology, by contrast, yields a nearly pure CO₂ flue gas with minimal separation effort, but has not reached commercial maturity due to unfavorable economics. The economic viability of this concept is highly sensitive to prevailing carbon pricing. According to the combustion reaction of NG with oxgen

CH4+2 O2→CO2+2 H2O

MO2=2∙MCO2MWCO2∙MWO2=2∙1t44gmol∙32gmol=1.45t

at least 1.45 t O2 needs to be produced per 1 t CO2. At an associated energy demand of 200 kWh/t O2 [https://doi.org/10.1016/j.jclpro.2021.128476] and an assumed electricity price of 0.20 €/kWh (Germany [https://www.smard.de/page/en/topic-article/5892/216044/industrial-electricity-price-trends]) this leads to oxygen production costs of 58 €/t CO2, which is already almost as high as current carbon certificates. Aditionally, futher costs for and makes oxyfuel combustion economically not viable at current carbon prices.

In retrofit concepts (low temperature design) air-N2 is substituted by CO2 to moderate peak flame temperatures through flue gas recycle. In contrast, temperatures of >2000°C are reached in high temperature design [source], which is way above the material limitation of the OCMC reactor design. Furthermore, this concept doesn’t leverage the main innovation (electrical energy input) of the OCMC design and hence realization with other materials obvious, what this concept rules out for further investigation.

leads to higher flame temperatures and hence The OCMC reactor concept overcomes these constraints, leaving the SMR process unchanged while requiring an air separation unit (ASU) instead of a carbon capture unit – at the cost of increased specific electricity demand.. Was dafür spricht, dass die tchnologie nicht primär an den materiallimitationen scheitert sondern tendenziell an den generellen OPEX economics mit zu teurer O2 Produktion und die OCMC Technologie entsprechend keinen untersuchenswerten Unterschied macht. Höhere Flammtemperaturen werden durch einen flue gas recycle gemanaged und sind keine technische Limitierung. Vielleicht auch unterschied zwischen neudesign und retrofitting. Aber Berretta meinte Temperatur wäre das Problem.

Abatement Effectiveness=1∆HR(T)ηfired energy, conventinal ∙Combustion Emissions=14.7MWht CH40.85∙4416t CO2t CH4=0.09t CO2MWh

Abatement Effectiveness=∆HR(850°C)ηfired energy, conventinal LHVNG∙Emission Factor∆HR(1100°C)=∆HR(850°C)0.8537MJm3 NG∙0.2778∙10-3MWhMJ0.79∙10-3t NGm3 NG∙57t CO2TJ∙10-6TJMJ∙37MJm3 NG0.79∙10-3t NGm3 NG∆HR(1100°C)=215.1kJmol CH4 reacted0.8513.01 MWht NG∙2.67t CO2t NG220.3kJmol CH4 reacted=0t CO2MWh

Since eSMR produces near-zero direct CO₂ emissions, a fair comparison requires an equally low-carbon reference – namely, conventional SMR equipped with flue gas carbon capture.

In the eSMR process, carbon is removed from a CO₂-rich stream downstream of the shift reactor at high pressures (pre-combustion capture). Assuming complete conversion in both the reformer and shift reactor, the maximum achievable CO₂ content is 20%, as dictated by the combined SMR and WGS stoichiometry:
CH4+2 H2O→CO2+4 H2

xCO2,max=11+4=20%

This CO₂ concentration is notably higher than the maximal ~12% CO₂ content of a stoichiometric CH4 combustion flue gas at atmospheric pressure (post-combustion capture), which is even lower in case of excess air:

CH4+2 O2+2∙0.790.21N2→CO2+2 H2O+7.52 N2

xCO2,max=11+7.52=11.7%

From the associated CO2 partial pressures follows that carbon capture integrated into the eSMR concept is associated with lower effort and thus cost than flue gas carbon capture [Quelle bezieht sich auf Stromproduktion und ist nach der partial pressure ausführung eigentlich nicht mehr nötig: https://papers.ssrn.com/sol3/papers.cfm?abstract_id=4284525].

As CO2 loading capacity of chemical and physical absorption solvents cross at around 10 bar CO2 partial pressure – the most important thermodynamic driver for capture – it is below that partial pressure, where typically chemical absorption is preferred, while above physical absorption gets of more interest. From this can be concluded that upon a pressure of ~50 bar (and hence for both processes) a chemical solvent system is used.

For quantification purposes an analysis of energy penalty estimates for CO2 capture in electricity production from natural gas via different capture-combustion modes is used, due to limited literature on capturing in SMR. Pre-combustion syngas dabei hat sogar größeren CO2 gehalt, während post-combustion sogar geringeren CO2 gehalt hat als in SMR process bei welchem noch das stoichiometric process CO2 inkludiert ist. Damit ist es eine sehr konvervative betrachtung zugunsten des eSMR prozesses. (und selbst unter diesen reicht es nicht aus um gap zu überwinden)

In SMR muss sehr viel mehr CO2 (cobustion +stoichiometric process emissions) gecaptured werden als in eSMR (nur stoichiometric process emissions),

This indicates a potential edge against the conventional process with CCUS, but is likely not to change the negative cost dynamic of higher utility costs.

As CO2 loading capacity of chemical and physical absorption solvents cross at around 10 bar CO2 partial pressure – the most important thermodynamic driver for capture efficiency – it is below that partial pressure, where typically chemical absorption is preferred, while above physical absorption gets of more interest. From this can be concluded that upon a pressure of ~50 bar (and hence for both processes) a chemical solvent system is used. For pressures above that consideration of different methods becomes relevant in a detailed techno-economic comparison.

Damit kommt die untersuchung zu dem Ergebnis, dass der konventinelle SMR-Prozess inlikely to replace ist, insbeondere aufgrund der höheren Elektrizitätskosten, welche bei einem verhältnismäßig geringen carbon price nicht durch den Vorteil einer besseren environmental bilanz refinanziert werden können. Der fairere Vergleich ist entsprechend mit einem konventinellen SMR Prozess inklusive post-combustion CCS. Hierbei stellt sich insbesondere die Frage ob die unterschiedlichen CO2-Partialdrücke und die damit verbundenen unterschiedlichen capture technologien den electricity economics nachteil wett machen. Es zeigt scih ganz klar, dass pre-combustion capturing eine bessere efficienz zeigt als der post-combustion process, dieser effekt verstärkt sich nochmal weiter, wenn man es mit einem post-combustion process vergleicht, welcher nicht die process CO2 emissionen enthält sondern ein reines CH4 combustion flue gas capturen muss (wie es bspw. Im ammoniasyngas prozess der fall ist). Darüber hinaus muss erwähnt werden, dass das konventinelle Reactorkonzept aufgrund von Materiallimitierugnen bei 40bar begrenzt ist. Dieser Vorteil stellt einen nicht von der hand zu weisendes Potential dar, da NG typischerweise mit 70 bar aus der pipeline kommt und synthesegas mit increased pressure in downstreamprozessen benötogt wird, eine Verdichtung nach der Refomierung jedoch aufgunrd der molzahlincreasing reaktion energieaufwendiger ist als davor. Der effekt ist jedoch geringer als die anderen analysierten effekte. Es wird als sinnig gesehen einen Prozess zu untersuchen, welcher genau diese Vorteil noch stärker nutzt. Naheliegend dafür ist der Ammoniasynthesegas prozess, da das reformierte synthesegasprodukt  für die Ammoniasynthese auf mindestens 150bar verdichtet werden muss. Zusätzlich kann in diesem Prozess der Vorteil des neuen Reaktorsystems von hohen Reaktionstemperaturen (>1000°C) bei der gleichzeitigen möglichkeit eines externen Energieeintrags (konträr zu adiabaten Reaktorkonzepten, welche sonst für solch hohe Reaktionstemperaturen genutzt werden) genutzt werden. Dies ist verbunden mit einer potentiellen CAPEX reduktion, bei einem ansonsten nahezu unveränderten Prozessdesign mit bereits kommerzielalisierten Technologien. Der Einsatz des neuen Reaktorkonzepts in diesem Prozess wird als vielversprechender eingestuft als im SMR-Prozess.

Mittels einer ausführlcihen TEA können der Einfluss von

- günstigerem capturing

- geringeren Verdichtungskosten

- und geringeren CAPEX-Kosten

gegenüebr der höheren Energiekosten von Elektrizität vs. NG-fuel bewertet werden.

Außerdem gibt es bereits zahlreiche Quellen, welche den eSMR prozess in technoökonomischen Analysen untersucht haben und zu einem negativen ergebnis gekommen sind. Dabei ist zwar der Druck ein geringerer, als der, welcher mit dem OCMC system gefahren werden könnte, aber dieser aspekt ist not likely to change the fundamental dynamics. Gleichzeitig gibt es wenige untersuchungen zu Ammoniaksynthesegasproduktion, außerdem inkjludiert diese auch den Aspekt des höheren Druckes, weshalb zumindest qualitative Rückschlüsse auf den einfluss eines höheren druckes im hydrogen prozess getroffen werden können und eine abschätzung getroffen werden kann ob eine detailanalyse evtl. Doch interessant wäre.

IEA electricity price estimates for the United States and China are based on industrial prices in Texas and Inner Mongolia, respectively – both among the lowest-cost regions within their respective countries. Reported values should therefore be interpreted as indicative lower bounds rather than national averages.

100% renewable electricity, ideal heat integration and no losses in eSMR; losses of 15% in conventional SMR, feed: pure methane, steam (S/C = 2,5), conventional process parameters: 850°C, 17 bar, OCMC process parameters: 1100°C, 51 bar, neglected: compression